# 实验五：航点修改与路径再规划

## 实验目的

本实验旨在掌握飞行任务执行过程中对关键航点进行更新的方法，使飞行路径能够随任务需求变化而调整；并掌握通过 MAVROS 重新推送航点至飞控系统与设置当前航点的流程，实现任务的在线更新与连续执行。理解无人机任务中路径再规划的重要性，并通过代码实现航点修改与任务路径更新。

## 实验说明

本实验主要涉及飞控任务中的路径规划与航点修改。学生将通过修改既定航点序列中的关键航点坐标，并对相关航点进行必要的几何约束修正，生成更新后的航点序列， 确保无人机能够按照新的路径执行任务。 实验将基于 waypoint_cal.cpp 文件，学生将学会如何在给定的航点序列中修改指定的航点（例如投弹点），并重新计算下一航点的坐标。完成该实验后，学生将能够使用 MAVROS 推送修改后的航点数据，确保飞控系统能够执行新的飞行任务。能够根据目标位置修改航点，并通过计算得到新的航点位置。确保推送的航点数据正确，并能设置当前航点以开始执行新任务。学生能够体会到在实际飞行任务中，如何应对目标位置变化并调整飞行路径。

## 实验步骤

步骤一：拉取飞控侧任务航点

使用 mavros_msgs::WaypointPull 服务，waypoint_pull_client 服务客户端对象从飞控系统拉取当前航点列表， 确认航点数据已经成功获取。此步骤写在 waypointSet 函数里面，此函数 1s 执行一次检查。通过判断是否到达 1 号航点，到达 1 号航点则拉取一次航线，并设置标志位表示已拉取航线。再通过订阅

// 回调函数，当无人机到达一个航点时调用 void Waypoint::waypointReachedCallback(const mavros_msgs::WaypointReached::ConstPtr& msg)

{

waypoint_seq.timestamp = formatRosTime(msg->header.stamp);	//读取到的时间戳

waypoint_seq.wp_seq = msg->wp_seq; waypoint_seq_buf.push_back(waypoint_seq); ROS_INFO("Waypoint reached: seq %d", waypoint_seq.wp_seq);

}

/mavros/mission/waypoints 话题获取航点列表赋值给变量 waypoints_read_vector，并把航线上预设的 10 号点经纬度坐标给 Lon_Scope 和 Lat_Scope 以及预设投弹点 drop_coordinates_lat，到达航点与航点列表函数：

//航点列表回调函数

void waypoints_callback(const mavros_msgs::WaypointList::ConstPtr& msg)

{

ROS_INFO("Received %lu waypoints.", msg->waypoints.size());

waypoints_read_vector = msg->waypoints;

for (const auto& waypoint : waypoints_read_vector)

{

//航点列表信息

ROS_INFO("	 Frame: %d", waypoint.frame); ROS_INFO("	 Command: %d", waypoint.command); ROS_INFO("	 Current: %d", waypoint.is_current); ROS_INFO("	Autocontinue: %d", waypoint.autocontinue); ROS_INFO("	 Param1: %f", waypoint.param1); ROS_INFO("	 Param2: %f", waypoint.param2); ROS_INFO("	 Param3: %f", waypoint.param3); ROS_INFO("	 Param4: %f", waypoint.param4); ROS_INFO("	 X: %f", waypoint.x_lat);

ROS_INFO("	Y: %f", waypoint.y_long); ROS_INFO("	 Z: %f", waypoint.z_alt);

}

//目标航点经纬度

Lat_Scope = int32_t(waypoints_read_vector[10].x_lat * 10000000); Lon_Scope = int32_t(waypoints_read_vector[10].y_long * 10000000);

//投弹坐标经纬度

drop_coordinates_lat_int = int32_t(waypoints_read_vector[10].x_lat * 10000000);

drop_coordinates_lon_int = int32_t(waypoints_read_vector[10].y_long * 10000000);

ROS_INFO("Lat_Scope: %d, Lon_Scope: %d", Lat_Scope, Lon_Scope);

}

步骤二：修改航点经纬度坐标数据

获取目标位置坐标后，修改投弹点经纬度坐标，将 waypoints_read_vector 赋值给 waypoints_send_vector，将计算出来的投弹点航线坐标 drop_coordinates_lat赋值给 waypoints_send_vector 相应点（即 10 号点）坐标，并根据给出的点的结构体 Point 和两点之间距离函数编写计算延长线坐标函数，根据投弹点和投弹前一个点的坐标，从而计算延长线上投弹点下一个点的坐标(即已知 9 号和 10 号点

坐标，求 11 号点坐标)。更新 waypoints_send_vector 相应航点（即 10 号点和 11

号点）的坐标。修改航点函数代码框架：

// 定义一个结构体来表示点

struct Point {

double x, y;

};

// 函数来计算两点之间的距离

double distance(const Point& p1, const Point& p2) {

return sqrt((p2.x - p1.x) * (p2.x - p1.x) + (p2.y - p1.y) * (p2.y - p1.y));

}

void changeWaypoint(void){ int drop_point = 10;

/*****************************************************/

//修改航点代码由学生编写

/*****************************************************/

}

步骤三：推送修改后的航点列表数据并设置当前航点

使用 mavros_msgs::WaypointPush 服务，wp_push_client 服务客户端对象将更新的航点数据发送给飞控系统，确保飞控系统能够接收到并更新数据。使用 mavros_msgs::WaypointSetCurrent 服务，wp_set_current_client 服务客户端对象设置当前航点，确保飞控能根据新的航点开始执行航线任务（即设置 8 号点为当前

航点，飞机将立刻飞往 8 号点，并执行 8 号点后续航线任务）。此步骤通知设置标志位使其只执行一次。

步骤四：验证航点修改

在 Mission Planner 中检查航点列表，确认修改后的航点已成功推送。确保修改后的航点数据能够被飞控系统正确接收，参考实验二，可通过后续实际飞行验证路径修改的效果。

把在虚拟机里写好的程序coordinate_cal 包放进RK3566 的catkin_ws 文件夹下面的 src 里面，编译，运行。

// 函数来计算延长线上特定距离的点坐标,已知 p1,p2 坐标，求延长线上点 p3

的坐标

Point calculate_extended_point(const Point& p1, const Point& p2, double distance) {

double dx = p2.x - p1.x; double dy = p2.y - p1.y;

参考代码：

double dist = ::distance(p1, p2);

// 计算延长线上点的比例

double ratio = (dist + distance) / dist;

// 计算延长线上点的坐标

Point p3;

p3.x = p1.x + dx * ratio; p3.y = p1.y + dy * ratio;

return p3;

}

//航点修改函数

void changeWaypoint(void){ int drop_point = 10;

waypoints_send_vector[drop_point].frame = mavros_msgs::Waypoint::FRAME_GLOBAL_REL_ALT;	//相对起飞点的坐标系

waypoints_send_vector[drop_point].command = 16;	// 航点 waypoints_send_vector[drop_point].is_current = false; waypoints_send_vector[drop_point].autocontinue = true; waypoints_send_vector[drop_point].param1 = 0;

waypoints_send_vector[drop_point].param2 = 0;

waypoints_send_vector[drop_point].param3 = 0;

waypoints_send_vector[drop_point].param4 = 0; waypoints_send_vector[drop_point].x_lat = drop_coordinates_lat; waypoints_send_vector[drop_point].y_long = drop_coordinates_lon; waypoints_send_vector[drop_point].z_alt = 20;

Point point1 =

{waypoints_send_vector[drop_point-1].x_lat ,waypoints_send_vector[drop_point

-1].y_long };

Point point2 =

{waypoints_send_vector[drop_point].x_lat ,waypoints_send_vector[drop_point]. y_long };

Point point3 = calculate_extended_point(point1, point2, 0.0004);

//推送修改后的 11 号航点 waypoints_send_vector[drop_point+1].x_lat = point3.x; waypoints_send_vector[drop_point+1].y_long = point3.y;

}